| Short Title | Finger Prick Blood Sampling by a Medically Trained Technician |
| --- | --- |
| Effective Date | 2 November 2011 |
| Approved by REB | 2 November 2011 |
| Version Number | 1 |

A finger prick blood sample will be obtained from each participant by pricking a finger using a  single-use disposable, retractable lancet (BD- Genie Lancet Cat. No 366583).These devices are used routinely by diabetics to test blood glucose. The BD microcollection Lancet ensures minimum discomfort to the participant, and will provide only an appropriate drop of blood for the analysis.

- The medical technician will wash hands thoroughly and put on gloves.

- The participant will be asked to sit or lie down, as per their personal preference.

- A blood collection site will be selected.

- The participant’s hand will be warmed with a heating pad or warm, moist towel for 3 minutes, to increase blood flow to the fingers.

- The participant will be asked to hold their hand downward to help increase blood flow toward the sampling site.

- The middle or ring finger (preferably) will be cleaned with an alcohol wipe and allowed to air dry to provide effective disinfection and prevent hemolysis.

- The technician will place the lancet device firmly on the chosen puncture site, with thumb on the activation button.

- The lancet will be used according to Becton Dickinson microcollection system specifications ( see attached).

- The participant’s hand will be gently massaged toward puncture site to obtain required drop of blood.

- Once a large drop of blood has formed (in less than a 1 minute period), it will be drawn into a micro-capillary tube provided with the Cholestech autoanalyzer kit, and applied to the Cholestech cartridge.

- If necessary, the technician will collect a second drop of blood which forms at the same site.

- The medical technician will wipe the site and then apply pressure to the puncture site using sterile gauze.

-The participant will be instructed to apply pressure to the site until bleeding has stopped. The technician will then apply a bandaid to the site.

-The blood sample will be loaded onto the Cholestech autoanalyzer cartridge,, according to the manufacturer’s instructions.

- The lancet device, and capillary syringe will be placed in the sharps container.

- All used materials, as well as the used Cholestech cartridges, will be disposed in a biohazard bag.

- Both biohazard bag and container will then be placed in the Medical Waste Management bin for removal by Environmental Health and Safety, for subsequent incineration.